SA2A to SA2A MKII – Upgrade Mod - 2017

You will need to buy the following parts

Transformers:

1 Sowter 1149 input transformer – sideleads – www.sowter.co.uk

1 Sowter 1010 output transformer – open frame – www.sowter.co.uk

Capacitors:

1 Solen Fast 10uf – 630v output capacitor – www.tubedepot.com

1 Silver Mica Capacitor, 120pf – 630V – www.mouser.com

Step 1)

Replace Cinemag input transformer with Sowter 1449 input transformer on to the main PCB where you read ALT/IN/TRAFO

Solder the green wire to point number 1

Solder grey wire to point number 2 on the main PCB

Solder the remaining wires to the output XLR PCB (CZ1)

White wire to pin 1

Red wire to pin 2

Blue wire to pin 3

Step 2)

Replace the Cinemag output transformer for the Sowter 1010 output transformer

Main PCB

Solder the black wire to the output on the main PCB

Output XLR PCB (CZ3)

Solder the grey wire to pin number 2

Solder the white wire to pin number 3

Step 3

Replace C25_ALT

Remove the Vishay Sprague output capacitor and solder the Solen Fast output capacitor

Step 4)

Remove C9 150pf – 500V capacitor

Solder the silver mica 270pf -500V onto C9.

Enjoy!

Step 5)

Calibration